# Digital Laboratory Lab08 Report

(Experiment Record Template)

FPGA Experiment-801: Tri-color LED Light Show

Create a 6-color gradient transition which is satisfied the following requirements:

- Modeling style: behavior

- Input: FPGA built-in CMOS oscillator (i.e., pin E3)

- Output: one built-in tri-color LED

- Brightness and color should be controlled by PWM signals. A duty cycle of 0 to 50% is recommended to avoid generating uncomfortable levels of brightness. Over 50% duty cycle may cause discomfort.

- Light show pattern (Duty cycle change list)

Note: The relationship between six colors.

sample-801 https://youtu.be/L5Jh26ULEEs

### Experiment Data

#### RTL Schematic

#### Simulation results

(Design your own testbench)

Notice that the duty cycles of the three different pwm signals are 0 at separate intervals. In practice, this should give us the correct color changing effect.

#### Practical results on the FPGA board

static record (photo, screen capture) or dynamic record (video link)

https://youtu.be/4SJtnICU4k8

OPTIONAL: design process

Let’s look at my code in three sections.

- PWM module

PWM stands for pulse width modulation. Simply put, this module changes the duty cycle of the clock to whatever percentage we require. My “pwm_generator” has 2 inputs (clk: the clk whose duty cycle you want to change, duty_cycle: the duty cycle you wish to change to) and 1 output (pwm: the new clk signal with modified duty cycle).

In the first always block, the 8-bit-reg “threshold” is set to 256 * “duty_cycle” / 100. 256 is the maximum value of an 8-bit-reg. The “duty_cycle” / 100 is to convert our input into percentage. For example, I’ll give “duty_cycle” a value of 25 if I want a duty cycle of 25% for my new modified pwm clk. The reason this is done in an always block is that “duty_cycle” may not stay constant. In fact, in this problem, we have to let our duty cycle count down from 50 to 0 and then back to 50 to create the gradient change in LED color. We will get into that part in detail later on.

In the second always block, I let reg “counter” start counting from 0 and rest back to 0 once it reaches 255. When its value is lower than “threshold”, it will send out 1. When its value is above the threshold, it will send out 0. This allows us to change the duty cycle of the clk.

- LED_control (Top Module) Part 1

My method here requires each color to have its own pwm signal, that’s why I have 3 8-bit-regs(dutycycleR, dutycycleG, dutycycleB) and three wires(pwmR, pwmG, pwmB). There are three control options for changing the duty cycles of the pwm signals. In these three regs (“directionR”, “directionG”, “directionB”) 0 is for counting down, 1 is for counting up, and 2 is for pause. The first always block is used to define these options.

- LED_control (Top Module) Part 2

How do I know when to change between counting up, down, or pause? Please refer to the following picture for further clarification. As you can see, whenever the duty cycle counters (dutycycleR, dutycycleG, dutycycleB) have to change direction, there’s always one of them who has a value of 50. In the always block below, I used this characteristic to control when and how the counting should change. Lastly, I assigned “pwmR”, “pwmG”, and “pwmB” each to a bit in a 3-bit-reg named “LED”, which is linked to the FPGA’s tricolor LED ports.

FPGA Experiment-802: Traffic Light

Create a traffic light with count-down timer which is satisfied the following requirements:

- Modeling style: behavior

- Input: FPGA built-in CMOS oscillator (i.e., pin E3)

- Output: one built-in tri-color LED, 2-digit 7-seg display

- Traffic light changing rules are as follows:

i. The LED color starts from red, and the timer counts down from 10 to 1.

ii. When the timer reaches 0, the LED color changes to yellow, and the timer counts down from 5 to 1.

iii. When the timer reaches 0 again, the LED color changes to green, and the timer counts down from 10 to 1.

iv. When the timer reaches 0 for the third time, repeat step i.

sample-802 https://youtu.be/Uajy9qdueZc

### Experiment Data

#### RTL Schematic

#### Simulation results

(Design your own testbench)

Take a look at the “LED” reg:

When “LED” = 4, that means its currently 100, which should make the FPGA’s tri-color LED glow red.

When “LED” = 6, that means its currently 110, which should make the FPGA’s tri-color LED glow yellow.

When “LED” = 2, that means its currently 010, which should make the FPGA’s tri-color LED glow green.

Additionally, notice how the “4” and “2” sections takes up around twice the time “6” does. This corresponds to how when the counter should count down from 10 to 0 when the light is red or green; the counter should count down from 5 to 0 when the light is yellow. The counter is also working as intended, you can see how it’s counting down correctly from 10 to 0 or 5 to 0.

#### Practical results on the FPGA board

static record (photo, screen capture) or dynamic record (video link)

https://youtu.be/C6ON80YtrZU

OPTIONAL: design process

Let’s look at this code in two parts:

- Basic:

I have two submodules in this top module, “FrequencyDiv” and “SevenSegment”. Here’s their purpose:

FrequencyDiv: simple frequency divider

SevenSegment: converts binary numbers into what we see on seven segment displays

The first always block here quickly toggles between enabling the tens digit seven seg and unit digit seven seg, allowing me to give them different outputs.

- Main section:

This is the most important part of my code. I have four conditions in this always block that triggers every second.

1st condition: If counter reaches 0 and the LED is currently red, set counter to 5 and LED color to yellow.

2nd condition: If counter reaches 0 and the LED is currently yellow, set counter to 10 and LED color to green.

3rd condition: If counter reaches 0 and the LED is currently green, set counter to 10 and LED color to red.

4th condition: If none of the above is true, simply count down by 1.

FPGA Experiment-803: Sequence Detector

Create a sequence detector which is satisfied the following requirements:

- Modeling style: behavior

- Input: Pmod input. Built-in CMOS oscillator (pin E3)

- Output: 8 LEDs

- Design a detection circuit that can detect the sequence 10110101.

- There is an LED indicating that the sequence has been detected.

- Use eight LEDs to display the currently captured data

### Experiment Data

#### RTL Schematic

#### Simulation results

(Design your own testbench)

As you can see, the “LED” output becomes 010 when “sequence” is 10110101 (the sequence we want to detect). In practice, it should make the tri-color LED on the FPGA green. For the rest of time, when “sequence” isn’t 10110101, “LED” stays at 100, telling the tri-color LED to glow red.

#### Practical results on the FPGA board

static record (photo, screen capture) or dynamic record (video link)

https://youtu.be/FyPHdcDFnFg

OPTIONAL: design process

My code here is fairly simple, there’s only 1 part:

I declared an 8-bit-reg called “sequence”, and it is connected to 8 of the LEDs at the bottom of the FPGA to indicate what the current sequence is. The input is passed in through “data”. I provided input test data using our Analog Discovery 2’s “Pattern” mode. New data will be passed in at a frequency of 1Hz. (one bit per second) In my always block, I used the left shift operator “<<” to shift “sequence” by 1 to the left and then assign the new input to the rightmost bit. The second always block tells the tri-color LED to glow green when the correct sequence is found or glow red otherwise.